Setup for ESC Workshop 1

Source Files

- Extract ESC_2007.zip to c:Actelprj\.  You should see four folders (Core_Console_project, HyperTerminal, Libero_projects and SoftConsole_workspace).

Folder Descriptions:

CoreConsole_project – contains a zip file with the CoreConsole project created in workshop 1.  You can extract this to the <drive>:\CoreConsole1.2.1\ConsoleDesigns folder if you have problems configuring the ARM7 system in CoreConsole.

HyperTerminal – contains a shortcut to the HyperTerminal application with settings configured.  Copy to your desktop.  You will use this for both workshops.

Libero_projects – contains the Libero projects for workshop 1.  VHDL and Verilog projects are provided.  There is also a completed project for each language.

SoftConsole_workspace – contains the SoftConsole project for workshop 2.  See Workshop2 setup for instructions on how to setup the SoftConsole project.

System Management Board Configuration

- Confirm the that jumpers on header J14 are configured as shown in the figure below:

- Confirm that the Tri-color LED jumpers (JP29, JP30 and JP31) are installed.

- Program the Fusion NVM with the STAPL file (NVM.stp) contained in the C:\Actelprj\ESC_2007\Libero_projects\NVM_programming_file folder.

Note: the folder also contains the .efc file for the NVM.  You can use that file when you generate the STAPL file for workshop 1 so that you don’t have to program twice.  For the workshop we will pre-program the NVM, so this step is not in the lab instructions.

- Connect the power supply and FlashPro3 programmer to the System Management board.

- Connect the serial cable between the RS-232 port on the System Management board and the serial port on your PC.

- Follow the instructions in the workshop 1 handout.

Setup for ESC Workshop 2

Fusion System Management board setup

- Program the Fusion FPGA array with the STAPL file (SysManTop3_24_07.stp) contained in the C:\Actelprj\ESC_2007\Libero_projects\workshop2_programming_file folder.

SoftConsole Setup

A SoftConsole project was extracted when you un-zipped ESC_2007.zip.  You need to import the project into SoftConsole and configure the debugger before you can do the exercises.

- Open SoftConsole and create a new workspace (File > Switch workspace).  Specify the following path in the Workspace launcher window:

C:\Actelprj\ESC_2007\SoftConsole_workspace

- Import the project into SoftConsole (File > Import).  Expand the path under General and select Existing Projects into workspace.

- Browse to folder C:\Actelprj\ESC_2007\SoftConsole_workspace\ESC_workshop2.

- Select ESC_workshop2 in the Import dialog box and click Finish.

- The files will be visible on the C\C++ Projects tab.  Confirm that there are no errors on the problems tab (you can ignore the 3 warnings that appear).

Configure the on-chip debugger

- From the Run menu, choose External Tools > External Tools.

- Choose Program in the Configurations window, right click and select New.

- Enter the following information on the Main tab of the Configurations window:

- Name: FlashPro3 On-Chip Debugging for CoreMP7

- Location: <SoftConsole Installation folder>\Arm\Bin\cliarm.exe

- Working Directory: <SoftConsole Installation folder>\Arm\Bin

- Arguments: initarm.tcl

- Click Apply then Close.

- Click Run > External Tools > Organize Favorites.

- Click Add in the Organize External tools dialog box.

- Select FlashPro3 On-Chip Debugging for CoreMP7 and click OK.  Then click OK in the Organize External Tools dialog box.

- Select Run > Debug.  Select Embedded Debug in the left window pane, right click and select New.

- Enter ESC_workshop2 in the Name field.

- Click the Browse button next to the Project field.  Select the project (ESC_workshop2) then click OK.

- Click the Search Project button next to the C/C++ Application field.  Select the project name then click OK.

- Click the Commands tab in the Debug window.  Enter the following lines of text in the Initialize window:

target remote :54545

load

- Enter the following line of text in the Run commands window:

cont

- Click Apply then click the Close button.

- Go to Window -> Preferences -> General -> Editors -> Text Editors -> Add a check mark in the “Show line numbers” box.

- Follow the instructions in the workshop 2 handout.