Q1.  Write T for true and F for false statements.						           	 {40}

- Pakistan’s 2nd Satellite, BADR-B was launched on 10 Dec 2001.				 [ T ]

- Ka band satellites work at 11.7-12.2 GHz and 14.0-14.5 GHz downlink/uplink frequencies. 	 [ F ]

- More than 60% of total satellites are meant for Communication services.			 [ T ]

- Collection of facilities, Users and Applications etc are called Space Segment.			 [ F ]

- Polar Orbits are also know as inclined orbits..							 [ F ]

- AsiaSat 3S, provides 24 C-band and 24 Ku-band transponders to the Asia-Pacific market.	 [ T ]

- S-DARS focus on broadcasting video programming without an audio element.		 [ F ]

- EIRP is a measure of the signal strength received on Earth from a satellite.			 [ T ]

- Tsys is the sum of To and the noise contribution of the receive antenna (Ta).			 [ T ]

- Free-space attenuation is determined by the inverse cube law.				 [ F ]

- LNB amplifies the satellite signals and down convert to a lower frequency.			 [ T ]

- The digital form of a signal is easier to transmit and is less susceptible to noise.		 [ T ]

- An optimally and fully loaded TDMA network can achieve 90% throughput.			 [ T ]

- Compared to GEO, LEO Satellites have High bandwidth, High power, High latency.		 [ F ]

- On 16 July 1990, Pakistan launched its first experimental satellite, BADR-I.			 [ T ]

- Geostationary Satellites remain above the equator at a height of about 22300 km.	 	 [ F ]

- MEO satellites are from 5000 to 15000 km above the earth.					 [ T ]

- LEO satellites are from 2000 to 4000 km above earth.						 [ F ]

- MEO satellites have an orbital period of 3 hours.						 [ F ]

- Van Allen radiation belts occur at ~2000-4000 km and ~13000-25000 km.			 [ T ]

- LEO satellites travel across the sky from horizon to horizon in 5 - 15 minutes.		 [ T ]

- In MEO, 56 satellites are needed to cover the entire earth.					 [ F ]

- HEO stands for High Earth Orbits.								 [ F ]

- Molniya and Tundra are examples of HEO satellites.						 [ T ]

- GEO satellites are the largest in number, orbiting the earth.					 [ F ]

- 3 GEO satellites 180° apart are sufficient to cover the entire earth.				 [ F ]

- A bent pipe repeater repeats the uplinked data without any alteration or recovery. 		 [ T ]

- In FDMA users can transmit on the same frequency but not at the same time	.		 [ F ]

- In CDMA multiple users transmit on the same frequency at the same time. 			 [ T ]

- Multiple access schemes are not required in networks that involve multiple users.		 [ F ]

- VSATs are example of Customer Premises Earth stations					 [ T ]

- Measat 1 provides service to Malaysia and throughout South East Asia.		 	 [ T ]

- First satellite - the Russian Sputnik 01 was launched in 1959.					 [ F ]

- ALOHA, allows stations to transmit immediately upon need resulting in minimum delays.	 [ T ]

- DSSS and FHSS are two forms of CDMA that are applied in practice.			 [ T ]

- First living creature was sent in space by Sputnik 01.						 [ F ]

- Ionospheric effects are effects of Faraday rotation, time delay, refraction, and dispersion.	 [ T ]

- DSSS has been applied in the Bluetooth wireless LAN standard.				 [ F ]

- Most CDMA applications over commercial satellites employ FHSS.				 [ F ]

- FDMA, TDMA, and CDMA will each continue to have a place in applications of the future.	 [ T ]

Note: Answer the following questions. (Attempt any Six questions.):

Q2. What is VSAT? How they are used for data communication and Internet services?	{10}

Q3. What are different classes of commercial satellites? Give example in each class?		{10}

Q4. What are different satellite application types? Explain over-the-air TV broadcasting? 	{10}

Q5. What are content delivery networks (CDNs)? Explain DARS and S-DARS?		{10}

Q6. How satellite links are designed? Explain Critical Elements of the Satellite Link?	{10}

Q7. Explain Ionospheric and Tropospheric atmospheric effects on Link Budget?		{10}

Q8. Why multiple access systems are used in satellites? How CDMA differs from the rest? 	{10}

The End