Q1.  Write T for true and F for false statements. (Attempt any Twenty.)		           	 {20}

- Konstantin Tsiolkovsky postulated the geostationary orbit.					 [ F ]

- Hermann Noordung first described the multi-stage rocket as means of achieving orbit. 	 [ F ]

- Arthur C. Clarke postulated the entire concept of international satellite telecommunication.	 [ T ]

- First satellite - the Russian Sputnik 01 was launched in 1959.					 [ F ]

- First living creature was sent in space by Sputnik 01.						 [ F ]

- Pakistan’s First satellite, Rehbar-I was successfully launched in June 1965.			 [ F ]

- On 16 July 1990, Pakistan launched its first experimental satellite, BADR-I.			 [ T ]

- Pakistan’s 2nd Satellite, BADR-B was launched on 10 Dec 2001.				 [ T ]

- Paksat-1, was deployed at 38 degrees E orbital slot in December 2002				 [ T ]

- More than 60% of total satellites are meant for Communication services.			 [ T ]

- Geostationary Satellites remain above the equator at a height of about 22300 km.	 	 [ F ]

- Collection of facilities, Users and Applications etc are called Space Segment.			 [ F ]

- Ka band satellites work at 11.7-12.2 GHz and 14.0-14.5 GHz downlink/uplink frequencies. 	 [ F ]

- Polar Orbits are also know as inclined orbits..							 [ F ]

- Compared to GEO, LEO Satellites have High bandwidth, High power, High latency.		 [ F ]

- MEO satellites are from 5000 to 15000 km above the earth.					 [ T ]

- LEO satellites are from 2000 to 4000 km above earth.						 [ F ]

- MEO satellites have an orbital period of 3 hours.						 [ F ]

- Van Allen radiation belts occur at ~2000-4000 km and ~13000-25000 km.			 [ T ]

- LEO satellites travel across the sky from horizon to horizon in 5 - 15 minutes.		 [ T ]

- In MEO, 56 satellites are needed to cover the entire earth.					 [ F ]

- HEO stands for High Earth Orbits.								 [ F ]

- Molniya and Tundra are examples of HEO satellites.						 [ T ]

- GEO satellites are the largest in number, orbiting the earth.					 [ F ]

- 3 GEO satellites 180° apart are sufficient to cover the entire earth.				 [ F ]

Note: Answer the following questions. (Attempt any Eight.):

Q2. What is Intelsat and Eutelsat? Explain their working and Organizational structure.	{10}

Q3. What is motivation behind satellite communication? Enlist any 5 missions of satellites.  {10}

Q4. Explain the basic Satellite System Elements?							{10}

Q5. What is meant by Uplink and Downlink? Briefly explain Direct to User services?	{10}

Q6. Write note on Satellite Orbits? Briefly explain each category?				{10}

Q7. What is meant by Van Allen radiation belts? Where is it located around the earth?	{10}

Q8. What are the Advantages and disadvantages of Satellite Communication?		{10}

Q9. Explain when to use terrestrial and Satellite Communication?				{10}

Q10. What is Polarization? What types of polarizations are used in RF communication?	{10}

The End