Subject: Satellite Communication		Quiz #3	      Total Marks: 100;   Total Time: 60 Min.

Reg. No. _____________________		      Name: __________________________

Q1. Write T for True and F for False statements.				      {64}

- DTH systems are also called Direct Broadcast Satellite.		                         [ T ]

- In DTH systems, the CA segment, authorizes individual IRDs over the space segment.       [ T ]

- The uplink compression elements are contained in the broadcast center earth station.          [ T ]

- The DirectTV broadcast centers in USA is a very small DTH provider.		      [ F ]

- The DTH compression systems fall into JPEG and BMP compression categories.		      [ F ]

- In C-band satellites, the users have to contend with terrestrial interference.		      [ T ]

- Ku-band systems are subject to rain fade, which produces occasional outages.	      [ T ]

- Canada has a DTH platform operating on Sinosat.			            	      [ F ]

- S-DARS systems deliver high quality video transmission to the end users.	             	      [ F ]

- S-DARS have the range limitation of terrestrial FM radio broadcasting.	             	      [ F ]

- S-DARS have lower audio quality than terrestrial FM radio broadcasting.	             	      [ F ]

- S-DARS first began in Europe as a free service by EuroWorldSpace.		      [ F ]

- First use of dedicated satellite audio was by Muzak in 1970s.		            	      [ T ]

- WorldSpace is more suitable for mobile reception than XM or Sirius.		      [ F ]

- Today, there are about 20 million DTH Service subscribers in the United States.	      [ T ]

- Three classic architectures for IT networks are DTH, SDARS and VSATS.	            	      [ F ]

- The familiar RS-232 connector definition is a good example of the physical layer. 	      [ T ]

- Layer 6 is responsible for routing information from end to end within the network.	      [ F ]

- Today there are more than 600,000 two-way VSATs installed worldwide.	            	      [ T ]

- Typical telephone networks have a real throughput of about 40 Kbps or less.                       [ T ]

- VSAT networks have typical inbound throughputs of 128 Kbps to 2Mbps. 	                         [ T ]

- VSATs use TDMA or ALOHA as multiple access schemes to share the channel. 	      [ T ]

- DTH systems are designed to transmit TV programming to home-receivers.		      [ T ]

- In ALOHA, the transmissions are uncoordinated.		      [ T ]

- In an acceptable operation, only 9  in 10 ALOHA packets experience collision.	      [ F ]

- Organizations employ VSATs primarily as replacements for terrestrial data networks.	      [ T ]

- In FDMA, each station is allotted a fixed interval of time to transmit data.		      [ F ]

- Noah Samara created WorldSpace that introduced S-DARS in Africa.		      [ T ]

- OSI model's Layer 6 provides syntactic and semantic services to the application layer.	      [ T ]

- Transport layer provides another level of assurance that data  will reach the end user.	      [ T ]

- Layer 6 reestablishes the connection in the event when the layer 4 is interrupted.	      [ F ]

- Sirius Satellite Radio is a commercial radio broadcasting company from Thailand.	      [ F ]

Q2. Select the correct answer to the following statements.				       {36}

- (   Ground   )   segment can contain a large number of Earth stations.

a. Space		b. Ground	  	 c. Earth

- There are more than 50 commercial (    Satellite    ) operators in 25 different countries.

a. Studio		b. Satellite 		 c. TV

- The satellite operator contracts with one of the approximately 12 (   spacecraft   ) manufacturers in the world for many of the elements needed for implementation.

a. Dish Antenna 	b. Cable 		 c. Spacecraft

- (     VSATs     ) use inexpensive user terminals designed for unattended operation.

a. Set-Top Boxes	b. VSATs		 c. Earth Stations

- The (    System integrators   ) can operate portions or the entire network, including maintenance and repair of equipment.

a. End Users	b. Cable operators	 c. System Integrators

- As time passes, Communications payloads are (    increasing   ) in capability and power.

a. Decreasing	b. Increasing	 c. Fading

- In multi-beam (   Ka-Band   ) satellites, on-board digital processors are commonly used to improve the capacity and flexibility of the network.

a. VHF-Band	b. UHF-Band	 c. Ka-Band

- The (    Hybrid   ) satellites permit cross-band operation, with the uplink Earth station at one band (e.g., Ku-band) and the downlink at the other (e.g. C-band).

a. Cross-Link	b. Hybrid		 c. Crosstalk

- The delivery of the signal to millions of small dishes demands the highest (    EIRP   ).

a. Orbit height	b. Apogee		 c. EIRP

- The (   Coverage   ) Pattern of the satellite determines the addressable market and the flexibility of extending services.

a. Coverage	b. Data		 c. Signal

- For a constant transponder output power, the EIRP varies (    Inversely    ) with the beam area.

a. Directly		b. Inversely		 c. Randomly

- It is common practice to call a (   Repeater   ) a transponder and vice versa.

a. Dish		b. Repeater		 c. Bent-pipe

- (   TWTs  ) tend to have the highest efficiency and are appropriate for broadcasting and digital information distribution.

a. TWTs		b. VSATs		 c. DVBs

- (   Contingency  ) planning involves making arrangements for backup satellite capacity and succession when operating satellites reach end of life.

a. Succession	b. Future		 c. Contingency

- (   TV Standards  ) define the format and quality of video signals that are intended for viewing by the general public.

a. ISPs		b.VSAT providers	 c. TV Standards

- (   Origination   ) defines standards for the video format that the camera uses to present the video image.

a. Migration	b. Capturization	 c. Origination

- (    Television    ) represents approximately 70% of commercial communications satellite use.

a. SDAR		b. VSAT		 c. Television

- (    STB     ) was originally introduced to provide a convenient interface between the cable drop and the home entertainment system.

a. DVB		b. UTP		 c. STB